# Cider Log

Batch 1 – Flat & Dry 6/20/2014

1 gallon Lyman Orchards cider

¼ cup light brown sugar

OG not measured – estimate 1.050

Nottingham yeast, nutrient, pectic enzyme

Rack to secondary 7/13 (16 days),  SG = 1.000 @ 75 deg.

Bottled 7/18. Minor lees in secondary jug.

FG = 1.002, ABV approx 6.2%

Sweetened with 3 TBSP Xylo Sweet

Primed with 1/8 cup corn sugar, SG = 1.005

Yield 8-1/2 12 oz bottles

Refrigerated 8/1/2014 (after 14 days)

Taste results:

Cloudy in bottles. Good alcohol content. Not sweet enough. No carbonation. Dry, not much apple flavor.

Batch 2 – Cherry Icky 7/16/2014

1 gallon Lyman Orchards cider

¼ cup light brown sugar

OG 1.050 @ 71°F

Rehydrated Nottingham yeast, nutrient, pectic enzyme

Bubbles within 3 hrs. Krausen by next morning.

Racked to secondary 8/3 (18 days). Seems to be clearer than previous batch. Bubbling once per 30 seconds. FG = 0.996 @ 73°F.

Bottled 8/8, (5 days in secondary).

FG = 0.995, ABV = 7.2%

Sweetened with 3 TBSP Xylo Sweet. Added ½ TSP organic cherry concentrate.

Yield 9 bottles.

Carb’ed with Carbonator Cap, 20 PSI at room temp.

Taste results:

Overpowering cherry flavor. Mild carbonation, almost nil.

Taste is artificial.

Batch dumped – yucky.

Batch 3 – Good - 9/19/2014

1 gallon Lyman Orchards cider

O.G. = 1.042

¾ cup light brown sugar (1 cup loosely packed, mixed with warm water).

O.G. = 1.050 @ 68F

1 TSP Rehydrated Nottingham yeast.

Racked to secondary 10/3/14 (2 weeks) F.G. = 1.000

Note: Seems to have more lees than previous batches. Maybe from the sugar?

Bottled 10/10/14 (1 week in secondary). F.G. = 1.000

Sweetened with ½ cup FAJC to 1.008 @ 69F

Transferred into two 2 liter bottles pressurized with 30 PSI of CO2.

Bottled without chilling first.

Yield: 8 bottles plus PET test bottle (see below)

Taste results:

Still just a bit tart.

Mildly carbonated.

Excellent apple flavor.

Good ABV

Batch 3 Experiment in Bottle Carbonating – 10/10/2014

One 12 oz PET bottle from Batch 3 before adding FAJC, with Carbonation Monitor in the cap. Added 1 TSP Xylitol, ½ TSP corn sugar, 1 sugar cube (1/2 TSP).

10/27 = 17 days – gage starting to move off zero.

| DATE | PRESSURE | TEMPERATURE |
| --- | --- | --- |
| 11/1 | 3 PSI | 69-70 |
| 11/3 | 4 PSI |  |
| 11/6 | 6 PSI |  |
| 11/9 | 8 PSI | 74F |
| 11/10 | 8 PSI | 70F |
| 11/11 | 9 PSI | 71 |
| 11/22 | 15 PSI | 84F |
| 11/27 | 12 PSI | 68F |

Refrigerated 11/27, tasted 11/28.

Taste results: Mildly carbonated, too sweet. Artificial tasting (Xylitol?). Icky.

Batches 4 & 5 10/10/2014

2 Gallons started at the same time. One gallon force carbonated, the other bottle carbonated with Carmel Sauce.

Batch 4 – Not Bad - From Wal-Mart Juice – No Secondary

1 Gallon Apple & Eve Wal-Mart apple juice

O.G. = 1.044

1/8 cup FAJC, O.G. = 1.050

Nottingham yeast, not rehydrated

10/27 – Bottle from Primary (no secondary). Cider is very clear.

F.G. = 1.002 Primary still bubbling very slowly.

Add 3.5 oz. white sugar. S.G. = 1.012

Force carbonated with carbonator caps, chilled overnight.

Bottle yield 9 bottles.

Taste results:

OK. Not much apple flavor, sort of like champagne. Medium carbonation.

Sweet enough.

Mike G. says he tasted yeast?

Batch 5 – Caramel Apple, Naturally Carbonated

1 Gallon Apple & Eve Wal-Mart apple juice

O.G. = 1.044

1/8 cup FAJC, O.G. = 1.050

Nottingham yeast, not rehydrated

Transfer to secondary 11/4/14, S.G = 1.001

Added caramel / cinnamon sauce into secondary bottle 11/9/14.

Before sauce, SG = 1.000 @ 69F

After 1 cup of sauce SG = 1.012

Bottled 11/11/14 - Still bubbling slightly. Tastes of cinnamon.

Yield: 8 bottles + PET monitor.

With carbonation monitor:

11/22 (11 days) 19 PSI @ 80F (put on heat radiator)

11/23 25 PSI @ 75F – Removed from heat.

Refrigerated 11/26, tasted Thanksgiving 11/27

Taste results:

WAY over carbonated. Gushers on opening bottles.

Funky flavor. Possibly too sweet. Won’t do this again.

Batches 6 & 7 “Perfect” Cider - Started 11/29/14

2 gallons Lyman Orchards cider

No sugar

¼ cup Organic raisins per gallon

OG 1.050 @ 67°F

Rehydrated Nottingham yeast, nutrient, pectic enzyme

Carboy showing bubbles within 3 hrs. Krausen formed by next morning.

Racked to secondary 12/12 (13 days). SG = 1.006

Temperature mostly 67 F

Cider is still very cloudy.

Secondary jugs topped off with CO2.

Secondary volume = 7x16 oz. + 4 = 116 oz.

Batch 6 bottled 12/27 (15 days in secondary)

F.G. = 1.000, ABV = 6.55%

Add Old Orchard FAJC ¾ cup to 1.010 S.G.

PET Keg @ 25 PSI 69°F, shake 6 times. Shake 2 more times over next 4 hrs.

Bleed pressure, refrigerate overnight.

Bottled from PET keg, @ 13 PSI, 40°F – got foam with bottle wand and 10 ft ¼” hose.

Yield – 9 bottles.

Taste Results:

Good apple flavor. Could use a bit more sweetness. Low carbonation.

Batch 7 bottled 12/28 – Best so far.

F.G. = 1.000

Add 3 TBSP Xylo Sweet

¼ cup Seneca FAJC, SG = 1.010???

Diluted with ¾ cup of water, SG = 1.005.

NOTE: Did not measure SG after the Xylitol. The 1.010 reading could have been due to the sweetener.

Added scant ¼ TSP Nature’s Flavors organic cider concentrate.

Yield = 9 bottles, including PET carbonation monitor.

Moved 3 bottles to refrigerator 1/7/15 (17 days). Pressure = 17 PSI @ 59°F = 2.1 Vols.

Taste results: Perfect sweetness, nice apple. Mildly carbonated, could use more (for my tastes).

One bottle was given to Fey. Monday Jan 12.

Carbonation Experiments

First experiment – using caramel sauce for priming:

12 oz water

Caramel sauce to S.G. 1.040

¼ TSP Nottingham yeast

½ TSP nutrient

After 21 hrs, pressure = 30 PSI @ 72°F = 2.4 volumes CO2

Sink water 147°F

Soaked for 20 minutes – 52 PSI @ 125°F

Bottom of PET bottle “popped” out.

Second Experiment – with caramel sauce (SG not measured):

12 oz water

½ TSP Nottingham lees, pinch of nutrient

20 PSI @ 69.2°F

Immersed into sink water @ 131°F

10 minutes, 42 PSI @ 123 F

Still rising after 17 min

Cracked bottle cap, bled out to 35 PSI @ 114 F

Immersed bottle into 180 F water, PET bottle popped bottom (not sure when).

Third Experiment – 12/2 – 12/13/2014

12 oz water

¼ TSP nutrient

½ TSP Nottingham lees

½ TSP corn sugar

After 12 days, 21 PSI @ 68 F

Immersed into hot sink water, 133°F

5 minutes, 29 PSI

10 minutes, 32 PSI

Transferred to 170°F pot after 15 min

5 minutes, bottle bulging 165°F, 35 PSI

Pressure drops when bottle pops –

Fourth experiment – with glass bottle: 1/3/15

2 TSP table sugar in 12 oz. water, SG = 1.010

22 PSI @ 65°F 1/8/15, should be 2.21 Vols

Immersed in hot sink water @122°F for 10 min, 45 PSI

Changed out the water, brought temp up to 136° for 7 minutes,

Gage reached 55 PSI and was still climbing. Calculated pressure for 2.21 Vols @ 136°F is 64.3 PSI.

Note: Calculated pressure for 2.0 volumes at 180 = 90 PSI.

After cooling to 67°F pressure fell to 25 PSI, which is equal to 2.31 volumes. It seems exposure to heat accelerated carbonation.

Glass bottle opened, perfect carbonation SG = 1.010

Batch 8A, 8B – 3 gallons started 1/19/2015

3 gallons Lyman Orchards cider

No sugar

OG 1.049 - 1.050

½ packet Safale S-04 yeast not rehydrated. Used nutrient, pectic enzyme

First use of 3 gallon Better Bottles carboy. Krausen formed by next morning.

Tested SG on 1/30/15 (11 days) 1.020 @ 66°F.

Checked 2/6/15 (18 days) 1.010 @ 65°F

Racked to secondary 2/8 (20 days). SG = 1.010

Yield 2 full gallons + about 1 quart

Note: Lees is powdery with this yeast. Need to wait after moving bottles. The 3 gallon carboy seems to lose a lot more than separate 1 gallons do.

Checked SG 2/23 Monday, 1.002 and still bubbling

Batch 8A – 3/1/15 Sunday (41 days total, 21 days in secondary)

Typical small batch, nothing special.

F.G. = 1.000, ABV = 6.5%

Bottled 1 gallon + remainder of quart from ½ gal jug (144 oz)

3/8 Cup Seneca FAJC to 1.005

3 TBSP Xylitol to 1.010

1/8 TSP acid blend

Yield: 12 bottles

Bottles carbonated to 30 PSI @ 69°F, 2.52 Volumes of CO2.

Taste: Good aroma and apple, slightly over sweet. Brother says best ever. Perfect carbonation.

Batch 8B   3/1/15  Best Yet!

SG = 1.000. Rack to tertiary on 1/8 cup of strong black tea and 1 crushed Campden tablet. Used some of the quart jug to top off the carboy.

Bottled 4/11 Saturday 41 days tertiary + 3 weeks secondary = 62 days.

F.G. Added FAJC to 1.010

¼ tsp Nature’s Flavors apple cider concentrate

Yield: 11 bottles

Carbed to 30 psi @ 70F

Taste: Excellent. Add more tea next time?

Batch 9 Started 4/26/15 – Regular and cherry versions

3 gallons Orchard cider

Nottingham yeast + nutrient

Ran out of pectic enzyme – pitched without it

S.G. 1.050

5/9 added yeast nutrient, S.G. = 1.010 (2 weeks in)

5/15 (3 weeks) racked to secondary @ 1.002. Not clear yet, maybe because of late addition of enzyme.

Added 1 cup of cherry puree to 1 gallon. SG + 6 points, to 1.008. Color is nice cherry.

Batch 9A, 5/31

Bottled with ¼ tsp concentrate

< ¼ tsp acid blend

FAJC to 1.010

Carbonated to 2.6 volumes, took 2 weeks +

Taste: OK. Not much apple flavor but drinkable. I think S-04 is better.

Batch 9B Cherry

5/31 racked to tertiary. Full gallon with a bit from batch A. SG = 1.002

Bottled 8/9  - 9 weeks tertiary, from 5/15 secondary = 11 weeks total

1.002 FG sweetened to 1.014 with FAJC

¼ tsp acid blend, ¼ tsp apple concentrate

Taste: Really good. Lots of alcohol. Cherry flavor is not noticeable except on the “burp”.

First Mead, 7/3/2015

JAOM Recipe

3 lb (1 quart) wildflower honey + about 8 oz Golden Blossom from grocery store.

3 quarts – 6.6 oz  Poland Springs water. Could do with 10 oz less

O.G. = 1.127

1 big cinnamon stick

2 cloves

1 large orange cut into about 12 pcs

Small handful of raisins

1 pkg bread yeast

Cleared and fruit fell in 65 days. Racked to secondary 9/12, exactly 4 x 750 ml bottles worth.

FG = 1.012, ABV = 15%

Bottled to 3 x 750 ml and 2 x 12 oz on 9/19, 75 days.

Taste: Cloves are overpowering. Needs time to age. Will taste again on Thanksgiving.

Update: Thanksgiving (2 months aging) MUCH better. Clove is mellowed, some orange is apparent. Drank 2 of the 3 bottles, everyone liked it. Last bottle probably be drank at Christmas.

Batch 10 Oct 4, 2015

6 gallons from Gotta’s Orchard in Portland

Used a 6 gallon bucket, plus a 1 gal carboy

OG 1.050

¼ TSP K-Meta in 5 gal, 1 crushed Campden tab in 1 gal.

+12 hrs added pectic enzyme

+ 32 hrs pitched dry S-04 ale yeast & nutrient

At pitch + 24 hrs no bubbles in airlock

Rehydrated more yeast, got airlock activity next day

Oct. 10, Sat (6 days) 1 gallon jug is 1.028, 5 gallon is 1.035

Oct 17 (13 days) 1 gallon is 1.010, 5 gallons is 1.018

Oct 24 (3 weeks) racked to secondary.

5 gallon bucket is 1.008, 1 gallon is 1.000

Split into 3 secondaries:

Batch 10A 3 gallons, add 1 cup black tea

Batch 10B 1 gal – ½ cup black tea, 12 oz frozen mixed berries

Batch 10C 1 gal – ½ cup tea, 1 cinnamon stick

Nov 8 (2 weeks in secondary)

10A Racked to 2-1/2 gallons + top-up for berry batch

9/1/16 Kegged with FAJC to 1.012.

Ecxellent. Bottled 5 bottles, sumbit 2 for NEHB Comp 9/25/16

10B Racked off berries to tertiary, FG = 1.000

10B Update 4/17/2016 – Racked to cannonball keg. Sweetened with frozen berry concentrate (Wildberry Punch) to 1.010.

Taste Notes: Very nice. Berries noticeable on the finish. Not much color, would not topup next time. One of the best batches.

10C Bottled 11/8 – 3 TBSP Xylitol, 0.189 oz corn sugar, ½ tsp cider flavor, carbonated to 25 PSI for Christmas (not ready for Thanksgiving). Has sediment in bottles.

Taste: Not bad. 9 bottles consumed for Christmas

Batch 11 Nov 28

5 gallons Gotta’s cider, OG 1.050

K-Meta, Pectic Enzyme

Rehydrated 71B-1122 with Go-Ferm, pitched 11/30

Yeast nutrient

Racked to secondary 12/6 (6 days) 1.006

3 gallons + 1 gallon

12/26 Saturday 20 days in secondary:

Batch 11A 3 gallons racked to tertiary with 3 Campden tabs. FG 1.000

9/25/16 Kegged 1 gallon plus 1/2 gallon from batch 10. FAJC to 1.015. Was 0.996

Racked other 2 gallons to carboys.

Batch 11B About 3 qt from 1 gallon, the balance was top-off  for 11A

Turning this into a cyser. Added ½ cup orange blossom honey + ½ TSP nutrient.

S.G. = 1.012

12/28/15 (2 days) added another ½ cup of honey. S.G. = 1.024. Some lees falling but airlock activity is very slow.

12/31/15 still 1.024 SG. Added another ½ cup OB honey, SG 1.040

Moved to cat food shelf (warmer)

1/16/16 Added 0.5 gr. Fermaid-O

Airlock picked up activity by next day

1/30 SG 1.012 still bubbling. Added another ½ cup honey (12 points?)

Total sugar addition about 48 points

2/13/16 – 47 days – bubbling 1 per minute. SG 1.005 – about 12% ABV

2/21/16 – Cold Crashed at 35F

2/27/16 – Racked to secondary. Yield about 3 quarts, FG 1.002

Topped up with 1 qt. from batch 10A. ABV should be about 10%

Added ½ oz. Medium + American Oak cubes floating on top, not

suspended in center. Still some cloudiness (6 days cold not enough)

3/20/16 – Cyser has cleared perfectly. Some lees in gallon jug.

3/26/16 – Racked to cannonball keg. FG 1.000. Added ½ cup Orange Blossom honey, FG about 1.015 (might not be mixed well yet). Purged CO2 tank 10 times with 25 PSI @ 67°F. Kegged at 14 PSI, 40°F after 3 days.

4/9/16 – Bottled after sampling from the keg. Not much left :P

Taste Notes: Really good. Gave 1 away to Fey, 2 to brother Jim. Good carbonation from keg. Nice sweetness, lots of alcohol but not harsh.

JAO BOMM 1 gallon Sat 12/5

3.5 lb orange blossom honey + a bit of wildflower

Peels and fruit of 1 small orange (removed pith)

25 raisins

½ cup dried currants

The stem of 1 clove

½ cinnamon stick

1/8 whole nutmeg

1 allspice berry

¾ TSP Potassium Bicarbonate (Should be 1/6 TSP!)

Smack Pack Wyeast 1388 smacked overnight

1 TBSP Fermaid-O

O.G. 1.132 Day 0, Sat 12/5

Day 2 = 1.11

Day 3 = 1.10

Day 7 = 1.082 added 1 TBSP Fermaid-O

Day 14 = 1.040, added 1 vanilla bean and airlock

Day 22 – Racked to secondary, still bubbling slowly, FG 1.025

Day 24 – Put outdoors in the cold, approx 35°F

Day 38 (1/11/16) – Added pectic enzyme

Day 43 – Added 1/5th package of SuperKleer (big mistake)

Cleared in 4 days, tastes and smells medicinal. Yuck.

Day 58 (1/31/16) Racked to ½ gallon

Cold crashed for 2 weeks and bottled – yield 2 wine bottles. SuperKleer sediment minor.

Drank 1 bottle with friends at Brothers, 2/10, 2 months from start. Tastes good, but missing orange. SuperKleer did that.

Batch 12A-B Cysers 11/09/2016

2 Gallons sweet cider from Beardsley’s Cider Mill

12A – 2 lb Orange Blossom honey from Stop & Shop OG 1.100

71B-1122 yeast + nutrient

Rehydrated, no Go-Ferm

12B - < 2 lb “Golden Blossom”, Orange Blossom, Sage Buckwheat, White Clover

OG 1.100

Cotes des Blanc yeast, rehydrated.

11/12/16 –

SG 12A = 1.072

SG 12B = 1.075

Aerate (shake), add 0.75 gm. Fermaid-0

Topped up with reserved must.

Added spices:

12A = 1 cinnamon stick, 1 clove, ¼ nutmeg

12B = 1 vanilla bean, 3 allspice berries, ¼ nutmeg

Moved to basement.

11/16/16 –

Batch 12A (71B) SG = 1.044

Batch 12B (CDB) SG = 1.045

Stirred.

11/20 – Moved upstairs. Swished daily.

12/10/16 = 4 weeks.

Batch 12A SG = 1.000

Batch 12B SG = 0.998

Removed spices from 12B. Has some floaters remaining (nutmeg?)

Spice bag broke on 12A. Got cinnamon out but clove and nutmeg fell in.

Both taste tart with some spice evident. Honey not prevailing.

12/27/16 – Moved 12A / 71B jug to fridge at 40°F

12/30/16 – Seems to be clearing. Lowered temp to 35.

Batch 13 Cider 11/13/2016

Beardsley’s “Hard Cider” blend, 6 gallons.

30% Winesap

30% Fuji

20% Granny Smith

20% McIntosh

4 bushels crab apples

1 bushel Quince

1 bushel Aronia berries

SG = 1.054, pH = 3.50, TA = 0.5%

Add 100 ppm (1/2 tsp) sulfite

11/14 7 am added pectic enzyme

11/14 9 pm racked off pectin mud

Pitched S-04 rehydrated in Go-Ferm about 11 pm

Temp = 68°F

11/15/16 Added 5 gm DAP

Moved to basement.

11/20/16 SG = 1.015

11/21/16 Racked to secondary, 3 gallons + 1 Gallon + ½ gallon

12/3/2016 in basement –